|  | ESP8266 | ESP-01 | ESP32 |
| --- | --- | --- | --- |
| File | http://arduino.esp8266.com/stable/package_esp8266com_index.json | http://arduino.esp8266.com/stable/package_esp8266com_index.json | https://dl.espressif.com/dl/package_esp32_index.json |
| Agregue libreria | ESP8266 | ESP8266 | ESP32 |
| PLACA | NodeMCU 1.0 ESP-12E Module | Generic ESP Module | DOIT ESP32 DEVKIT V1 |
| Speed |  | 11520 |  |

|  | Placa: NodeMCU 1.0 ESP-12E Module az-delivery-devkit-v4 o FireBeetle-ESP32 En la parte plateada dice el modelo de microcontrolador Es de tipo ESP8266 |
| --- | --- |

|  | En la parte de atras de placa En la placa escoja : - ESP32 Dev Module / ESP32 DOIT DEVKIT V1 - ESP32 Dev Module / NodeMCU 1.0 ESP-12E Module |
| --- | --- |

| DELANTE | ATRAS |
| --- | --- |
| DOIT MODEL ESP8266MOD | 9600 BAUD DRIVER CH3406 |

| File / Preferences |  |
| --- | --- |

https://dl.espressif.com/dl/package_esp32_index.json, http://arduino.esp8266.com/stable/package_esp8266com_index.json

Search for ESP32 / ESP8266

and press install button for the “ESP32 by Espressif Systems“:

Select your Board in Tools > Board menu

- ESP32 Dev Module / ESP32 DOIT DEVKIT V1

- ESP32 Dev Module / NodeMCU 1.0 ESP-12E Module

Select the Port (if you don’t see the COM Port in your Arduino IDE, you need to install the CP210x USB to UART Bridge VCP Drivers):

https://www.silabs.com/products/development-tools/software/usb-to-uart-bridge-vcp-drivers

3. Open the following example under File > Examples > WiFi (ESP32) > WiFiScan

4. Press the Upload button in the Arduino IDE. Wait a few seconds while the code compiles and uploads to your board.

6. If everything went as expected, you should see a “Done uploading.” message.

7. Open the Arduino IDE Serial Monitor at a baud rate of 115200:

| 8. Press the ESP32 on-board Enable button and you should see the networks available near your ESP32: |  |
| --- | --- |

## Troubleshooting

Port not enabled

En menu escoge Tools y el Puerto aparece en gris, no aperece COM1, COM2, o COM3,

Solucion:

- Verifique en Control Panel / Devices que esta instalado el Driver

- Instale el driver de link CP210x USB to UART Bridge VCP Drivers):

- DOWNLOAD  ch340 driver generic

Time Out

If you try to upload a new sketch to your ESP32 and you get this error message

“A fatal error occurred: Failed to connect to ESP32: Timed out… Connecting…“.

It means that your ESP32 is not in flashing/uploading mode.

Having the right board name and COM por selected, follow these steps:

| Hold-down the “BOOT” button in your ESP32 board |  |
| --- | --- |
| Press the “Upload” button in the Arduino IDE to upload your sketch: |  |
| After you see the “Connecting….” message in your Arduino IDE, release the finger from the “BOOT” button |  |
| After that, you should see the “Done uploading” message. Now press Enable-Reset Button to restart ESP32 |  |

You’ll also have to repeat that button sequence every time you want to upload a new sketch. But if you want to solve this issue once for all without the need to press the BOOT button, follow the suggestions in the next guide:

- [SOLVED] Failed to connect to ESP32: Timed out waiting for packet header

If you experience any problems or issues with your ESP32, take a look at our in-depth ESP32 Troubleshooting Guide.

## Wrapping Up

Take a look at the ESP32 troubleshooting guide.

- Learn ESP32 with Arduino IDE [eBook + Video Course]

- ESP32 vs ESP8266 – Pros and Cons

- Free ESP32 Projects and Tutorials

- Build an ESP32 Web Server with Arduino IDE

- ESP32 DHT11/DHT22 Web Server with Arduino ID

My personal experience: in 2021, I use almost exclusively the ESP32 for IoT projects. It is more versatile, and it comes with much more functionalities like Bluetooth, different wake-up sources, many peripherals,

Once you move to the ESP32, you won’t want to go back to the ESP8266.

- The ESP32 is faster than the ESP8266;

- The ESP32 comes with more GPIOs with multiple functions;

- The ESP32 supports analog measurements on 18 channels (analog-enabled pins) versus just one 10-bit ADC pin on the ESP8266;

- The ESP32 supports Bluetooth while the ESP8266 doesn’t;

- The ESP32 is dual-core, and the ESP8266 is single core;

- The ESP8266 is cheaper than the ESP32;

- The ESP8266 has a wider community (although we don’t think that at this point, the difference is that big);

- For many IoT and Wi-Fi projects, the ESP8266 can do the job for a lower price;

- Both boards support MicroPython firmware.